# Project Update

Project: Reinforcement Learning for Smart HVAC Control Using CityLearn

Date: April 2026

## 1. Current Status

Core prototype is complete and runnable end-to-end.

Environment setup on Windows has been stabilized using a dedicated conda environment (citylearn-rl).

CityLearn schema loading, custom reward injection, observation configuration, and single-building filtering are fully implemented.

Three RL algorithms are implemented: SAC, PPO, and TD3; plus a Rule-Based Controller (RBC) baseline.

Training/evaluation scripts and an integrated pipeline script are available (tune -> train -> evaluate).

## 2. Environment Definition / Problem Setup

Environment: CityLearn v2.5.0 dataset citylearn_challenge_2023_phase_2_local_evaluation.

Control scope: currently single-building (Building_1), with optional multi-building mode.

Action space: continuous central-agent action vector over active devices (cooling storage, electrical storage, cooling device).

State space: expanded to include time context, weather, selected forecasts, indoor temperature, storage SOC, load, pricing, carbon intensity, and net electricity consumption.

Reward function: custom ComfortEnergyReward with three components: energy penalty + comfort penalty + carbon penalty.

Comfort penalty uses a +-1.0 C comfort band around setpoint.

## 3. Methods Tried

Rule-Based Controller (RBC) baseline using time-of-use heuristics.

SAC (off-policy) with replay buffer and entropy-based exploration.

PPO (on-policy) with episodic rollout alignment (n_steps = episode length).

TD3 (off-policy deterministic) with twin critics and delayed policy updates.

Optional hyperparameter tuning using Optuna.

## 4. Preliminary Results

Latest evaluation run (normalized CityLearn KPIs; lower is better for most metrics):

RBC baseline: electricity_consumption_total = 1.3889, cost_total = 1.2496, carbon_emissions_total = 1.3378, discomfort_proportion = 0.7812.

SAC: electricity_consumption_total = 0.6500, cost_total = 0.6119, carbon_emissions_total = 0.6420, discomfort_proportion = 0.4656.

PPO: electricity_consumption_total = 0.3864, cost_total = 0.3623, carbon_emissions_total = 0.3845, discomfort_proportion = 0.9804.

Interpretation: PPO currently achieves the best energy/cost/carbon reductions, while SAC provides a better comfort-energy tradeoff.

Plots and artifacts are being generated in results/, including KPI comparison, reward curves, and temperature traces.

## 5. Challenges and Fixes

Dependency conflicts on Windows (openstudio/gymnasium/torch DLL) were resolved with a dedicated conda workflow and pinned versions.

CityLearn schema dict loading initially failed; fixed by reading schema from CityLearn cache path and setting root_directory.

Building API differences (LSTMDynamicsBuilding missing observation_names) were handled by robust helper functions.

Action formatting for central_agent mode was corrected (single flattened action list expected).

Temperature trace plotting initially failed due to missing setpoint history on some building objects; now handled via robust setpoint extraction and fallback plotting.

## 6. Next Steps

Run longer training (more episodes) for SAC/PPO/TD3 and compare stability over multiple seeds.

Use Optuna results to update final training hyperparameters in config.py.

Evaluate TD3 in the same KPI table and include it in final comparison plots/reports.

Test multi-building mode for generalization and broader system-level performance.

Prepare final presentation narrative around trade-offs: energy/cost/carbon vs thermal comfort.

## 7. Summary

The project has progressed from a minimal SAC prototype to a more complete experimental framework with multiple RL algorithms, custom reward engineering (including carbon), robust environment tooling, and reproducible training/evaluation pipelines. The remaining work is mainly performance tuning, broader experiments, and final report packaging.