Project objective and current direction

We will implement an RL-based HVAC controller in CityLearn for a single building, train it on selected state variables such as indoor temperature, outdoor temperature, time information, and electricity-related features, and compare its performance against a simple rule-based controller. The purpose of this stage is to confirm that RL can learn a control policy that balances energy savings and thermal comfort better than a fixed heuristic approach.

Understanding of the environment

CityLearn is a simulation environment for building energy management and demand response. It models buildings and distributed energy resources such as heat pumps, electric heaters, batteries, and optionally PV generation. In this environment, the control agent interacts with the building step by step: it observes the current operating condition, takes a control action, and then receives the next state and a reward.

The environment is suitable for this project because it already provides:

- building energy dynamics,

- weather and calendar inputs,

- configurable observations and actions,

- built-in reward functions,

- and rule-based controllers that can be used as baselines.

Verified Against CityLearn 2.5.0 Docs

- Built-in dataset: citylearn_challenge_2023_phase_2_local_evaluation ships with CityLearn and requires no external download. It contains 3 buildings; we configure it for a single building at runtime by setting include: false on the others via schema dict manipulation.

- Action space: Continuous, per device, normalized to [-1, 1]. Each value controls storage charge (+) / discharge (−) or heat pump setpoint. This is native to CityLearn and most compatible with SB3.

- Wrappers: CityLearn 2.5.0 ships NormalizedObservationWrapper (min-max scales obs, cosine-encodes cyclical obs like hour) and StableBaselines3Wrapper (converts multi-building obs/actions to flat arrays). Both are required for SB3.

- central_agent=True is required for SB3 (SB3 does not support multi-agent).

- Schema loading: CityLearn's DataSet.get_schema() returns the schema as a Python dict, which we modify before passing to CityLearnEnv.

## RL Algorithm Choice: SAC (Stable Baselines3)

- SAC (Soft Actor-Critic) is the right choice because:

  - The action space is continuous → rules out DQN

  - Off-policy → more sample-efficient than PPO for energy simulations

  - Entropy-regularized → automatic exploration, no manual tuning of epsilon

  - SB3 SAC is officially demonstrated in CityLearn 2.5.0 docs with the exact wrappers we use

- PPO is a valid alternative (simpler, on-policy) but requires more timesteps

MDP formulation: state, action, reward, policy

State

The state is defined as the current operating condition of the building, including indoor temperature, outdoor temperature, time-of-day information, and electricity-demand-related features. This state representation is designed to capture both comfort conditions and the energy context needed for HVAC control.

- indoor temperature,

- outdoor temperature,

- hour of day,

- day type or weekday/weekend,

- current electricity demand or net electricity consumption,

- optionally predicted weather or price-related variables if available,

- optionally HVAC mode or previous action.

State (after NormalizedObservationWrapper, all in [0,1]):

- hour, day_type, month (cyclical, cosine-encoded)

- outdoor_dry_bulb_temperature

- indoor_dry_bulb_temperature

- indoor_dry_bulb_temperature_set_point

- net_electricity_consumption

- cooling_storage_soc (if building has cooling storage)

Action

The action represents the HVAC control decision at each time step. In the simplified version of the project, the agent will choose whether to increase, decrease, or maintain HVAC output, which can later be extended to a continuous control signal if the environment configuration supports it.

- increase heating / cooling output,

- decrease heating / cooling output,

- maintain current output,

or, if implement continuous control:

- choose a normalized HVAC power level between 0 and 1. Action: Continuous vector [-1, 1]; each element controls one device (cooling storage, electrical storage, etc.). CityLearn determines dimension from the schema.

Reward

A practical implementation is to begin with CityLearn’s default or comfort-related reward logic and then adapt it into a weighted reward that penalizes both net electricity consumption and deviation from temperature setpoint.

reward = -(α × net_electricity_consumption + β × comfort_penalty)

comfort_penalty = max(0, |T_indoor - T_setpoint| - comfort_band)

Default: α=0.5, β=0.5, comfort_band=1.0°C

Policy

The policy is the learned control strategy that determines which HVAC action should be taken under each building condition. During training, the agent updates this policy based on rewards received from the environment, with the goal of improving long-run comfort-adjusted energy performance.

Evaluation Success Criteria

CityLearn's env.evaluate() returns KPIs normalized to the baseline (1.0 = same as baseline). We report:

- electricity_consumption_total < 1.0 (RL uses less energy)

- discomfort_proportion < 1.0 (RL is more comfortable)

- cost_total < 1.0 (RL costs less)

A value below 1.0 on any metric = improvement over baseline.

SAC is chosen because the action space is continuous ([-1, 1] per device), so DQN is ruled out. SAC is off-policy with a replay buffer, making it far more sample-efficient than PPO for year-long simulations (8,760 steps per episode). Its entropy regularization handles exploration automatically.

Single-building setup is done at schema level: env_setup.py calls DataSet.get_schema() to get the built-in schema as a Python dict, sets include: false on every building except Building_1, then passes the modified dict to CityLearnEnv. No external files needed.

Custom reward subclasses CityLearn's RewardFunction base class and is referenced by module path in the schema dict, so CityLearn instantiates it natively — no extra gym wrapper needed on top.

Baseline Rule-Based Controller (RBC) charges cooling storage during off-peak hours (22:00–05:00) and discharges during peak hours (12:00–18:00). This is a common commercial building heuristic and gives a meaningful comparison point.